## Literature review

Literature regarding fuel consumption prediction has transitioned from traditional physical-based (white-box) models, such as the mean value phenomenological model constructed by Heywood (2019), which necessitate a comprehensive understanding of internal combustion engines' characteristics, such as intake system, delivery system, torque production system, and exhaust system, toward data-driven (black-box) approaches (Yang et al., 2022)[Paper 28]. This shift leverages advancements in machine learning, big data, and computational power to circumvent the limited scalability and adaptability of physical models to heterogeneous real-world complications due to their extensive calibration and detailed subsystem data requirements. Despite the superior predictive performance of the black-box models (Zhao et al., 2023)[Paper38], they often lack model interpretability, physical plausibility, and robustness.

The critical analysis of the existing fuel consumption literature is conducted through three conceptual pillars, which are modelling approaches, explainability approaches, and model robustness/generalisation under domain shift.

Most of the current studies concerned with the vehicles’ energy/fuel consumption construct data-driven models (Alam et al., 2025; Liu et al., 2025; Singh & Sharma, 2025; Vicinanza et al., 2025; Yoo et al., 2025; Zhang et al., 2025)[Papers 1, 3, 4, 29, 2, 12, 37]. While a smaller subset of the research focuses on the construction of constraint-aware learning approaches (Lei et al., 2024, 2025; Tran et al., 2024)[Papers 39, 40, and 42].

#### Data-driven learning approach

High-frequency telemetric data dominate data-driven learning approaches to predict instantaneous fuel or energy consumption. Singh & Sharma (2025) [Paper 1] utilised Time-Series Transformers to forecast fuel usage and capture long-term temporal patterns in speed, acceleration, engine load, and road gradient signals, outperforming LSTM and GRU baseline models. Similarly, Zhang et al. (2025)[Paper 4] propose a dual-layer LSTM-Transformer (DLLT) for predicting real-time energy consumption and driving dynamics in plug-in hybrid electric vehicles (PHEV), achieving R2 values of 0.99 for fuel consumption, 0.86 for acceleration, and 0.81 for electric power. In the heavy-duty vehicles, Liu et al. (2025) [Paper 3] and Vicinanza et al. (2025) [Paper 29] utilise CatBoost and Feedforward Neural Networks (FFNN), respectively, to model the impact of real-time driving behaviour, engine load (e.g. RPM, torque) and vehicle weight on fuel rate. While highly accurate, these models typically rely on telemetric "operational" data (RPM, MAF, Throttle), making them unsuitable for the early design phase. These models are predominantly black-box or weakly interpretable and do not enforce hard physical constraints. In addition, these models have limited robustness and generalisability due to modelling dataset specificity to similar truck classes (diesel classes), data collected from a single province (Shandong)(Liu et al., 2025)[Paper 3], a single vehicle model (Honda Accord PHEV) (Zhang et al., 2025)[Paper 4], or one truck model (IVECO S-Way) (Vicinanza et al., 2025)[Paper 29].

A smaller but growing number of data-driven paradigms have explored using static vehicle design specifications such as engine displacement, curb weight, and regulatory test-cycle parameters rather than high-frequency operational telemetry to estimate fuel consumption (Yoo et al., 2025; Zahid & Jamil, 2026)[Paper 37, Paper 2] and CO2 emissions forecasting (Alam et al., 2025)[Paper12].

Yoo et al. (2025) [Paper2] developed a machine learning framework using approximately 953 vehicles from a Korean regulatory dataset and demonstrated that ensemble methods such as Random Forest and Extra Trees could capture nonlinear relationships between vehicle attributes and fuel efficiency, while SHAP and LIME provided post-hoc interpretability of influential factors. However, the dataset used exhibits substantial categorical imbalance and domain narrowness, including strong manufacturer dominance (Hyundai ≈60% and Kia ≈35%), a single-country context, reliance on a single regulatory standard, highly skewed transmission distribution (approximately 74% automatic vehicles), and limited vehicle category diversity (cars ≈96% versus vans ≈3%). Such imbalances increase the likelihood of manufacturer or category-specific overfitting and reduce confidence in cross-category or cross-market generalisability.  Similarly, Alam et al. (2025) [Paper 12] proposed a multilayer perceptron model (MLP) using Canadian government data, reporting high predictive accuracy and identifying fuel consumption variables, engine size, and cylinders as dominant predictors for CO₂ emissions through SHAP analysis. However, the inclusion of fuel consumption metrics as predictors introduces a strong mechanical linkage to the target variable, creating a target-proxy leakage risk that may inflate performance without demonstrating genuine predictive learning from design attributes.  Zahid & Jamil (2026) [Paper 37]  compared linear regression, support vector regression, and random forest models to predict combined fuel consumption using the Canadian government dataset spanning 2000 - 2024 and incorporated temporal validation to demonstrate performance stability across years. Even though the non-linear random forest model demonstrated superior predictive performance, the model remains unreliable and problematic due to the inclusion of city fuel consumption as a predictor for combined fuel consumption, because the combined rating is constructed directly from 55% city fuel consumption and 45% highway fuel consumption (Natural Resources Canada, 2025), which introduces strong target-proxy and target-leakage risks that explain the inflated predictive performance across all prediction models. Although the temporal holdout evaluation represents a step toward robustness assessment, the claimed generalisability remains limited because none of the models underwent systematic robustness testing, such as cross-manufacturer validation, cross-regulatory cycle evaluation, or feature perturbation experiments.

Regardless of using telemetric or design specification data, all data-driven models have several methodological limitations that remain consistent across studies. First, the models are purely data-driven and do not incorporate physical constraints or monotonicity enforcement, even though well-established engineering relationships with modelling predictors govern vehicle fuel consumption. Second, interpretability is predominantly post-hoc, meaning explanations describe correlations learned by black-box models rather than guaranteeing physically consistent behaviour. Third, robustness evaluation is limited, typically relying on random or temporal splits without systematic testing across manufacturers or regulatory cycles. Most importantly, several studies include predictors that are mechanically linked to the target variable, such as combined CO₂ emissions or city fuel consumption to predict combined fuel consumption, which introduces target-proxy leakage and can inflate performance metrics while reducing early-design usability.

#### Constrained learning approaches:

Physics-informed or theory-guided machine learning models have increasingly been proposed as a middle ground between purely physics-based simulators and purely data-driven predictors. In the automotive industry, the physical/constraint awareness is achieved through soft constraints (hybrid-loss) methods or strong constraints (physical equation-enforced) methods.

Under the hybrid loss / weakly constrained paradigm, Lei et al. (2024) [Paper 39] and Tran et al. (2024) [Paper 42] integrated the physical knowledge as learning guidance rather than strict enforcement. Lei et al. (2024) [Paper 39] proposed a hybrid powertrain modelling framework that combines physics-based engine and battery knowledge with data-driven residual learning (Transformer), and neural network correction components to improve prediction accuracy and optimisation outcome. Physical guidance during the learning process led to improved fuel consumption and thermal predictions compared with purely data-driven baselines. The proposed model remains largely of a black-box nature, with limited robustness and generalisability only to plug-in hybrid electric vehicle (PHEV) under specific driving scenarios (commuting) and a single regional context (Beijing). Similarly, Tran et al. (2024) [Paper 42] optimised electric vehicle design by incorporating the design geometries and their respective aerodynamics constraints through regularisation terms rather than strictly enforced physical equations within the autoencoder training process. While the proposed framework demonstrates accurate drag prediction and design optimisation capability in simulation environments, its interpretability remains limited, and robustness is constrained by reliance on a single simulation pipeline without real-world validation. Furthermore, the study focuses on aerodynamic performance rather than fuel consumption directly.

On the other hand, Lei et al. (2025) [Paper 40] introduces a theory-constrained neural network (TCNN) to model fuel cell electric truck (FCET) energy consumption. In this framework, physical equations derived from fuel-cell electrochemical principles are embedded directly into the model architecture through a theory-guided filtering mechanism, while subnetworks correspond to interpretable physical components such as voltage losses and temperature dynamics. The integration of domain knowledge improves predictive accuracy while maintaining theoretical consistency and enhanced modular interpretability compared with purely data-driven or weakly constrained frameworks. Nevertheless, the applicability of the framework remains restricted to a specific hydrogen fuel cell vehicle architecture (FCET) and high-frequency operational telemetry, limiting transferability to other vehicle types or propulsion systems (e.g., internal combustion or battery electric vehicles) without substantial redesign of physics-based components. Furthermore, robustness evaluation is primarily confined to internal model and architecture comparisons using data collected from a single vehicle type and geographic region (Hunan Province), with no systematic testing across geographic regions, climates, or temporal shifts.

Overall, (Lei et al., 2024, 2025; Tran et al., 2024) [Papers 39, 40, and 42] demonstrate the spectrum of physics integration in machine learning for automotive systems, ranging from soft physics guidance to strongly constrained architectures. However, common limitations persist, including dependence on telemetry or simulation data rather than early-design specifications, limited robustness validation under domain shift, and incomplete interpretability despite physics integration. These gaps highlight the need for constraint-aware machine learning approaches that combine constraint-awareness with robustness and applicability to early-stage vehicle design contexts.

## Research Gaps

Despite the high predictive power reported across all studies that approached vehicles’ energy/fuel consumption, five systematic gaps are identified:

- Target Leakage & Proximity Inflation: Several high-performing models (e.g., include features like CO2 emissions or City Fuel Rate to predict Combined Fuel Rate. Since these are mechanically linked by fixed regulatory equations, the resulting performance is artificially inflated and lacks true predictive utility.

- The Model Interpretability: Most studies rely on data-driven black-box architectures such as ensemble methods or Transformer-based networks, where interpretability is achieved primarily through post-hoc explanation techniques (e.g., SHAP, LIME) rather than intrinsic transparency.

- Physical Reliability: Modelling approaches demonstrate enhanced predictive accuracy, but they do not enforce physical constraints to maintain consistent relationships. Consequently, the learned models may produce statistically accurate yet physically implausible behaviours, undermining engineering reliability and regulatory trustworthiness.

- Early-Design Usability: A substantial portion of data-driven and constraint-aware modelling frameworks relies on telemetric operational data that is only available after a vehicle is built and driven. Such models are inherently unsuitable for early-stage vehicle design, where only static design specifications are available, particularly for manufacturers seeking pre-prototyping optimisation and regulators requiring early compliance estimation.

- Narrow Robustness: Models are often validated on single-region or single-vehicle datasets, failing to account for cross-country, cross-manufacturer, or climate-driven domain shifts.

## Study motivations

Understanding and predicting vehicle fuel consumption patterns is an interdisciplinary research domain where machine learning, regulatory compliance, automotive engineering, and environmental sustainability intersect. The global transportation sector's contribution to total CO₂ emissions of 16.2% makes developing countries adopt electric vehicles as an eco-friendly alternative to traditional Internal Combustion Engine (ICE) vehicles (Alam et al., 2025)[Paper 12]. The high cost of electric vehicle technologies and components (rare-earth elements) makes the adoption of electric vehicles in undeveloped or developing countries theoretical in the short term. Consequently, an accurate, physically plausible, transparent and robust prediction of ICE vehicles' fuel consumption is central to transport decarbonisation and regulatory compliance.

## Research Questions:

- To what extent does incorporating physical constraints improve the robustness and generalisation performance of fuel consumption prediction models?

- How does an intrinsically interpretable modelling approach compare to black-box ensemble models in terms of predictive performance and explanation stability?

- Can fuel consumption be reliably predicted using vehicle design specifications only, without reliance on operational telemetry data under fixed testing conditions?

- How robust is the proposed modelling framework under manufacturer hold-out and cross-regulatory validation settings?

## Positioning of the Present Study (Aim & Objectives)

Aim:

To design and evaluate an explainable, robust, and constraint-aware machine learning framework for predicting vehicle fuel consumption using vehicle design specifications.

Objectives:

- Develop a unified, machine-learning-ready dataset by integrating vehicle design specifications with corresponding fuel consumption records and deriving engineering-informed predictors.

- Build baseline (non-constraint-aware) machine learning models to estimate fuel consumption based on vehicle design characteristics.

- Develop explainable, constraint-aware modelling frameworks with varying levels of complexity and interpretability to ensure physically consistent predictions.

- Compare the predictive performance differences between constraint-aware and non-constraint-aware models.

- Evaluate the generalisability and robustness of the proposed frameworks under different validation scenarios.